SUMEC Sensor options

Distance measurement

# TF-LUNA

LiDAR Range Sensor / ToF

Distance measured: 8m

Communication protocol: UART/I2C

Output: Digital

Technology: Infrared

Input voltage: Vin = 5V

Connector: Molex PicoBlade

TF-LUNA Manual

PROS:

- We have 3 of them

- Low FoV: 2°

- Accurate

- We know they work

CONS:

- Cost (500/600Kc)

- Dimensions: 35mm*21.25mm*13.5mm

- Blind zone: 20cm – unreliable measurement

- Harder measurement for objects with high reflectivity

# Pololu Distance Sensor

LiDAR-based Distance Sensor / ToF

Distance measured: 50, 130cm

Communication: Digital Pulse Width

Output: Digital

Technology: Infrared laser

Input voltage: Vin = 3-5V

Connector: 2,54 - 3Pad holes

Polulu Pulse Width manual

PROS:

- Dimensions: (21.6 × 8.9 × 3.5 mm)

- Accurate

CONS:

- Cost: (400Kc)

- Minimum range: 4 cm (for accurate measurement); < 1 mm (for detection)

- High FoV: 15°

- We don’t know if they work

# VL53L1X

Laser distance measurement sensor / ToF

Distance measured: 400cm max

Communication: I2C

Output: Digital

Technology: Infrared laser

Input voltage: Vin = 3-5V

Connector: 2,54 - Pad holes

VL53L1X Datasheet

PROS:

- Cost: (200Kc)

- Dimensions: 29.5x22mm

- Accuracy 1mm

CONS:

- Programmable FoV from 15 to 27 degrees (?)

- We don’t know if they work

# VL53L3CX

Laser distance measurement sensor / ToF

Distance measured: 300cm max

Communication: I2C

Output: Digital

Technology: Infrared laser

Input voltage: Vin = 2.8-5V

Connector: 2,54 - Pad holes

VL53L3CX

PROS:

- Dimensions:

- True distance measurement independent of target size and reflectance

CONS:

- Cost: (400Kc)

- We don’t know if they work

# TF-LC02

LiDAR Distance Sensor / ToF

Distance measured: 130cm, Infrared, Vin = 3-5V, 3 Pad holes, Output: Digital Pulse Width

Polulu Pulse Width manual

PROS:

- Dimensions: (21.6 × 8.9 × 3.5 mm)

- Accurate

CONS:

- Minimum range: 4 cm (for accurate measurement); < 1 mm (for detection)

- Very High FoV: 27°

- Cost: (400Kc)

- We don’t know if they work