SUMEC Optics

1. Distance measurement

2. Presence of objects

# TF-LUNA 1.

LiDAR Range Sensor / ToF

Distance measured: 8m

Communication protocol: UART/I2C

Output: Digital

Technology: Infrared

Input voltage: Vin = 5V

Connector: Molex PicoBlade

TF-LUNA Manual

PROS:

- We have 3 of them

- Low FoV: 2°

- Accurate

- We know they work

CONS:

- Cost (500/600Kč)

- Dimensions: 35mm*21.25mm*13.5mm

- Blind zone: 20cm – unreliable measurement

- Harder measurement for objects with high reflectivity

# Pololu Distance Sensor 1.

LiDAR-based Distance Sensor / ToF

Distance measured: 50, 130cm

Communication: Digital Pulse Width

Output: Digital

Technology: Infrared laser

Input voltage: Vin = 3-5V

Connector: 2,54 - 3Pad holes

Polulu Pulse Width manual

PROS:

- Dimensions: (21.6 × 8.9 × 3.5 mm)

- Accurate

CONS:

- Cost: (400Kč)

- Minimum range: 4 cm (for accurate measurement); < 1 mm (for detection)

- High FoV: 15°

- We don’t know if they work

# VL53L1X 1.

Laser distance measurement sensor / ToF

Distance measured: 400cm max

Communication: I2C

Output: Digital

Technology: Infrared laser

Input voltage: Vin = 3-5V

Connector: 2,54 - Pad holes

VL53L1X Datasheet

PROS:

- Cost: (200Kč)

- Dimensions: 29.5x22mm

- Accuracy 1mm

CONS:

- Programmable FoV from 15 to 27 degrees (?)

- We don’t know if they work

# VL53L3CX 1.

Laser distance measurement sensor / ToF

Distance measured: 300cm max

Communication: I2C

Output: Digital

Technology: Infrared laser

Input voltage: Vin = 2.8-5V

Connector: 2,54 - Pad holes

VL53L3CX

PROS:

- Dimensions:

- True distance measurement independent of target size and reflectance

CONS:

- Cost: (400Kč)

- We don’t know if they work

# TF-LC02 1.

LiDAR laser distance Sensor / ToF

Distance measured: 200cm

Communication: UART

Output: Digital

Technology: Infrared laser

Input voltage: Vin = 3-3.7V

Connector: JST SH

Polulu Pulse Width manual

PROS:

- We have 1 of them

- Cost: (250Kč)

- Very small size 20 x 11.5 x 7.6 mm

- Accurate

CONS:

- Minimum range: 3 cm (for accurate measurement)

- High FoV: 18°

- We don’t know if they work

- Almost no documentation

# Pololu Digital Distance Sensor 2.

LiDAR based distance sensor / I/O

Distance measured: 50/100cm

Communication: HIGH/LOW

Output: Digital

Technology: Infrared laser

Input voltage: Vin = 3-5.5V			Distance seen is based on the jumper config.

Connector: 2.54, 3 Pad Holes

Polulu digital detection manual

PROS:

- Size 21.6 × 8.9 × 3.5 mm

- Accurate – 1mm

- Adjustable distance

- Adjustable type of measurement

CONS:

- Cost: (500Kč)

- FoV: 15°

- We don’t know if they work